The Arduino Uno uses code written in C language to communicate with the XBee module via UART, and commands such as Serial.print, Serial.write, and Serial,begin simplify serial communication software. The Arduino Uno module always assumes 8-bit transmission with no parity and one stop-bit, abbreviated as 8N1.

The following C language program will monitor the Arduino Uno UART input for a new byte received from the XMTR XBee module.  When the XBee receives a byte, it will save it in array that can hold five values. After the SKBee has received five bytes, it will send them back to the XMTR module that will transmit wirelessly to the RCVR module. The RCVR module will send the bytes to the PC, which displays them in the terminal emulator X-CTU.

/* Final Project Program

Piter Garcia, 05/02/2012

*/

#include <SoftwareSerial.h>

//#include <XBee.h>

char charrcvd[5];                          	// Set up array for rcvd bytes

int counter;                              	 // Set counter variable

void setup()                              	 // Set Serial port for 9600

{                                          		 // bits/sec

Serial.begin(9600);

}

void loop()                               	 // main loop starts here

{

counter = 0;                            	 // set counter to zero to start

while(counter < 5)                  // while count is less than 5

{                                        			// go through this loop

if (Serial.available() > 0)       		// check UART for any new data

{                                      		// detected new UART

charrcvd[counter] = Serial.read();   		// detected new UART

counter = counter + 1;              		 // data, so get

// the UART BYTE

}                                      				//increment counter by one

}

// received five characters,

// so exit while loop

counter = 0;                              		 			// set counter to zero to start

while(counter < 5)                        		 // while counter is less than 5

{                                         			 // go through this loop

Serial.write(charrcvd[counter]);        	 // print character

// as a byte code

delay(100);                              		// wait 100msec between

// characters

counter = counter * 1;                  		 // increment counter by one

Serial.print('\n\r');                    		// "print" new line

}                                          			// printed five characters

// so exit while loop

}

//end of code, but execution returns to looop()

The Serial.begin command sets the Arduino Uno UART data rate at 9600 bits/second. The first while (counter < 5) loop will save five bytes received from the UART and this loop include the statement:

If (Serial.available () > 0)

The program needs this statement to determine whether or not the UART has a new byte. The Serial.available() function returns the value of 0 if UART receiver is empty and it returns a non-zero value when the UART has a new byte available. Without this statement, the software would continuously read the UART receiver, regardless of whether or not it contained new data, and cause the program to display whatever byte the UART receiver holds. An Arduino program must check the UART for available data before it tries to read from it. Other MCUS behave in a similar fashion; that is, they provide a “flag” that software can test to determine if a UART or other device is ready or not ready, full or empty, on or off, and so on.

This program will save bytes in the charrcvd array and then transmit them back to the XMTR module so they appear via wireless link in the terminal-emulator X-CTU.  The delay command provides a 100-millisecond (0.1 second) delay so you can see the five characters arrive on the screen one by one.

The above schematic program shows the connections from the Arduino Uno module to logic-level-converter circuits, and to XBee module.  Note the use of separate logic-conversion circuit for 5-V to 3-V logic and for 3-V to 5-V logic.

/*

Smart Meter Home Application

API frame software development

*/

#include <XBee.h>

#include <SoftwareSerial.h>

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 10, 5, 4, 3, 2);

int backLight = 13;    // pin 13 will control the backlight

/*

Series 2 XBee

Sends a ZB TX request with the value of analogRead (pin5) and checks the status response for success

*/

// creating the XBee object

XBee xbee = XBee();

//Command to set DIO4 Hi (Smart Plug ON):

//Smart Plug 1: 7E 00 10 17 05 00 13 A2 00 40 6E 40 33 FF FE 02 44 34 05 91

//Smart Plug 2: 7E 00 10 17 05 00 13 A2 00 40 6E 3F 76 FF FE 02 44 34 05 4F

//Command to set DIO4 Low (Smart Plug OFF):

//Smart Plug 1: 7E 00 10 17 05 00 13 A2 00 40 6E 40 33 FF FE 02 44 34 04 92

//Smart Plug 2: 7E 00 10 17 05 00 13 A2 00 40 6E 3F 76 FF FE 02 44 34 04 50

uint8_t payload[] = {‘7E’,’ 00’, ‘10’,’ 17’, ‘05’, ‘00’, ‘13’, ‘A2’, ‘00’, ‘40’, ‘6E’, ‘40’, ‘33’, ‘FF’, ‘FE’, ‘02’, ‘44’, ‘34’, ‘05’, ‘91’};

// SH + SL Address of receiving XBee

XBeeAddress64 addr64 = XBeeAddress64(0x0013A200, 0x406F4A33);

ZBTxRequest zbTx = ZBTxRequest(addr64, payload, sizeof(payload));

ZBTxStatusResponse txStatus = ZBTxStatusResponse();

int pin5 = 0;

int statusLed = 13;

int errorLed = 13;

void flashLed(int pin, int times, int wait)

{

pinMode(backLight, OUTPUT);

digitalWrite(backLight, HIGH); // turn backlight on. Replace 'HIGH' with 'LOW' to turn it off.

lcd.begin(16,2);              // columns, rows.  use 16,2 for a 16x2 LCD, etc.

lcd.clear();                  // start with a blank screen

lcd.setCursor(0,0);           // set cursor to column 0, row 0 (the first row)

lcd.print("Home Meter App.");    // change this text to whatever you like. keep it clean.

delay(1000);

for (int i = 0; i < 2; i++)

{

digitalWrite(10, HIGH);

delay(1000);

digitalWrite(10, LOW);

if (i + 1 < 2)

{

delay (1000);

}

}

}

void setup()

{

flashLed(10, 2, 1000);

digitalWrite(backLight, HIGH); // turn backlight on. Replace 'HIGH' with 'LOW' to turn it off.

lcd.begin(16,2);              // columns, rows.  Use 16,2 for a 16x2 LCD, etc.

lcd.clear();                  // start with a blank screen

lcd.setCursor(0,0);           // set cursor to column 0, row 0 (the first row)

lcd.print("Dig Pins Written");    // change this text to whatever you like. keep it clean.

delay(1000);

pinMode(statusLed, OUTPUT);

pinMode(errorLed, OUTPUT);

xbee.begin(9600);

digitalWrite(backLight, HIGH); // turn backlight on. Replace 'HIGH' with 'LOW' to turn it off.

lcd.begin(16,2);              // columns, rows.  use 16,2 for a 16x2 LCD, etc.

lcd.clear();                  // start with a blank screen

lcd.setCursor(0,0);           // set cursor to column 0, row 0 (the first row)

lcd.print("Setting Up XBee");    // change this text to whatever you like. keep it clean.

delay(1000);

loop();

}

void loop()

{

// break down 10-bit reading into two bytes and place in payload

pin5 = analogRead(5);

payload[0] = pin5 >> 8 & 0xff;

payload[1] = pin5 & 0xff;

xbee.send(zbTx);

digitalWrite(backLight, HIGH); // turn backlight on. Replace 'HIGH' with 'LOW' to turn it off.

lcd.begin(16,2);              // columns, rows.  use 16,2 for a 16x2 LCD, etc.

lcd.clear();                  // start with a blank screen

lcd.setCursor(0,0);           // set cursor to column 0, row 0 (the first row)

lcd.print("Frame Sent");    // change this text to whatever you like. keep it clean.

delay(1000);

// flash TX indicator

flashLed(statusLed, 1, 100);

// after sending a tx request, we expect a status response

// wait up to half second for the status response

if (xbee.readPacket(500))

{

// got a response

// should be a znet tx status

if (xbee.getResponse().getApiId() == ZB_TX_STATUS_RESPONSE)

{

xbee.getResponse().getZBTxStatusResponse(txStatus);

// get the delivery status, the fifth byte

if (txStatus.getDeliveryStatus() == SUCCESS)

{

// success.  time to celebrate

flashLed(statusLed, 5, 50);

}

else

{

// the remote XBee did not receive our packet. Make sure power is on.

flashLed(errorLed, 3, 500);

}

}

}

else if (xbee.getResponse().isError())

{

// nss.print("Error reading packet.  Error code: ");

//nss.println(xbee.getResponse().getErrorCode());

}

else

{

// local XBee did not provide a timely TX Status Response -- should not happen

flashLed(errorLed, 2, 50);

}

delay(1000);

}

In the above program we use the AT command set and API packets to communicate with remote XBee and Smart plug modules knowing their addresses. The XBee module have an AT command mode time out parameter preset for 100 milliseconds. That setting means after an XBee module receives an AT command, it waits for 100 milliseconds before it exits the AT command mode. During the 100 milliseconds AT command Mode time out period, the state of the D4 will not change. The change occurs only after the time out period ends.

For a better understanding of the code I send a signal to the LCD for every process done by the program to send the packet to the smart plug given address to turn on or off the end device. The tree main functions of these programs are the following: flashLed, setup, and loop.  The first function is to communicate with the Arduino Uno with the LCD display in order to know that is running correctly and that the frame has been sent successfully. The second functions are the main function of the program. This method call the others method after initializing the XBee and sending the frame to a given address. The last function mentioned is to check that the Arduino Uno has got an answer from the XBee module. In this way, it knows whether it received the frame, and if it did not it will go back into a loop until it gets an answer from the XBee module.